Diploma in Aerospace System & Management

Final Year Project (EGK324) Report

Project Title: 
Development of an electronic device for kitchen fire prevention (Phase 4) & Development of an innovative massage bed

Project Supervisor: 
Seah Leng Wah

Project Student: 
Tan Jia Hao (211496M)

Enable notifications so that the phone can receive notifications whenever which requirements it hit (e.g gas level too high)

Example for the gas sensor, the code that is required is “Blynk.logEvent(“Gas-level-exceeded-threshold”);

# Code & Codes Explanation

#define BLYNK_TEMPLATE_ID "TMPL6o33TOgKF"

#define BLYNK_TEMPLATE_NAME "Quickstart Template"

#define BLYNK_AUTH_TOKEN "sk8-yXxalIHrOE6HhOVWyuNu7tZHkrJi"

#define BLYNK_PRINT Serial

#include <WiFi.h>

#include <WiFiClient.h>

#include <BlynkSimpleEsp32.h>

#include <Wire.h>

#include <Adafruit_MLX90614.h>

char ssid[] = "Jiahao";

char pass[] = "cardes123";

// Define pin numbers for various sensors and actuators

#define FLAME_SENSOR_1_PIN 39

#define FLAME_SENSOR_2_PIN 36

#define MOTION_SENSOR_PIN 26

#define GAS_SENSOR_PIN 34

#define BUZZER_PIN 13 // Define your buzzer pin number

#define GAS_THRESHOLD 3500 // Set your gas threshold value

#define TEMPERATURE_THRESHOLD 35.0 // Set your temperature threshold value

Adafruit_MLX90614 mlx = Adafruit_MLX90614();

BlynkTimer timer;

void setup() {

Serial.begin(115200);

// Initialize pin modes for sensors and actuators

pinMode(FLAME_SENSOR_1_PIN, INPUT);

pinMode(FLAME_SENSOR_2_PIN, INPUT);

pinMode(MOTION_SENSOR_PIN, INPUT);

pinMode(BUZZER_PIN, OUTPUT);

// Initialize the MLX90614 sensor

mlx.begin();

// Connect to Blynk using Wi-Fi

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass);

// Set up a timer to send sensor data to Blynk at regular intervals

timer.setInterval(1000L, sendSensorData);

}

void loop() {

// Run Blynk and timer events

Blynk.run();

timer.run();

}

void sendSensorData() {

// Read sensor values

int flame1 = digitalRead(FLAME_SENSOR_1_PIN);

int flame2 = digitalRead(FLAME_SENSOR_2_PIN);

int motion = digitalRead(MOTION_SENSOR_PIN);

int gasValue = analogRead(GAS_SENSOR_PIN);

// Invert flame sensor readings

flame1 = !flame1;

flame2 = !flame2;

// Read object temperature in Celsius

float temperature = mlx.readObjectTempC();

// Send sensor data to Blynk

Blynk.virtualWrite(V7, flame1);  // Flame Sensor 1

Blynk.virtualWrite(V8, flame2);  // Flame Sensor 2

Blynk.virtualWrite(V2, motion);  // Motion Sensor

Blynk.virtualWrite(V5, gasValue);  // Gas Sensor

Blynk.virtualWrite(V6, temperature);  // Temperature Sensor

// Check gas level and activate the buzzer if it exceeds the threshold

if (gasValue > GAS_THRESHOLD) {

digitalWrite(BUZZER_PIN, HIGH);

// Log gas level event for potential notification

Blynk.logEvent("Gas-level-exceeded-threshold");

} else {

digitalWrite(BUZZER_PIN, LOW);

}

// Check temperature and activate the buzzer if it exceeds the threshold

if (temperature > TEMPERATURE_THRESHOLD) {

digitalWrite(BUZZER_PIN, HIGH);

// Log temperature event for potential notification

Blynk.logEvent("Temperature-issue-detected");

}

// Check for flame detection and activate the buzzer

if (flame1 == 1 || flame2 == 1) {

digitalWrite(BUZZER_PIN, HIGH);

// Log flame detection event for potential notification

Blynk.logEvent("Flame-detected");

}

// You can add more logic here to trigger additional actions based on sensor readings

}

# Codes

// Blynk template information

#define BLYNK_TEMPLATE_ID "TMPL6CsmGhs8p"

#define BLYNK_TEMPLATE_NAME "Vibrator Testing"

#define BLYNK_AUTH_TOKEN "Im2fMtVY-5B9pCHmHzaMVYUBUbYM0qKG"

#define BLYNK_PRINT Serial

// Include necessary libraries

#include <WiFi.h>

#include <WiFiClient.h>

#include <BlynkSimpleEsp32.h>

#include <WiFiUdp.h>

#include <ArduinoOTA.h>

// Define GPIO pins for LEDs and Blynk virtual pins for sliders

#define LED1 23

#define LED2 22

#define LED3 21

#define VPIN_SLIDER_1 V0

#define VPIN_SLIDER_2 V1

#define VPIN_SLIDER_3 V2

// WiFi credentials

char ssid[] = "Jiahao";

char pass[] = "cardes123";

// Variables to store slider values

int slider1 = 0;

int slider2 = 0;

int slider3 = 0;

// Setup function

void setup()

{

Serial.begin(115200); // Initialize serial communication

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, pass); // Initialize Blynk with authentication token and WiFi credentials

ArduinoOTA.begin(); // Initialize ArduinoOTA for over-the-air updates

}

// Blynk callback for slider V0

BLYNK_WRITE(VPIN_SLIDER_1)

{

slider1 = param.asInt(); // Get the slider value from Blynk

analogWrite(LED1, slider1);

Serial.print("V0 Slider value is: ");

Serial.println(slider1);

}

// Blynk callback for slider V1

BLYNK_WRITE(VPIN_SLIDER_2)

{

slider2 = param.asInt(); // Get the slider value from Blynk

analogWrite(LED2, slider2);

Serial.print("V1 Slider value is: ");

Serial.println(slider2);

}

// Blynk callback for slider V2

BLYNK_WRITE(VPIN_SLIDER_3)

{

slider3 = param.asInt(); // Get the slider value from Blynk

analogWrite(LED3, slider3);

Serial.print("V2 Slider value is: ");

Serial.println(slider3);

}

// Loop function

void loop()

{

Blynk.run(); // Run Blynk

ArduinoOTA.handle(); // Handle ArduinoOTA updates

}